Deep Water – Gulf of America (DW-GOA)

confined space entry and work

| Project | DW-GOA |
| --- | --- |
| Document Title | Confined Space Entry and Work |
| Document Number | HSE0085 |
| DCAF Control ID Number |  |
| Document Revision | 0.1 |
| Document Status | Approved |
| Document Type | Procedure |
| Owner/Author | Corey Boice / Christopher Camardelle |
| Issue Date | September 2025 |
| Expiration Date | April 29, 2026 |
| ECCN | EAR 99 |
| Security Classification | Internal |
| Disclosure | None |
| Revision History shown on next page |  |

Revision History

| Revision | Date | Summary of Changes | Author |
| --- | --- | --- | --- |
| 0.0 |  |  |  |
| 0.1 | September 2025 | Update the document template to Standard Template Update GOM to GOA references Updated the GOM references in HSE0085-GL01 | Christopher Camardelle |

SUMMARY

This document covers confined space entry at DW-GOA facilities must follow this procedure to ensure safety from physical and atmospheric hazards, supplementing global regulations and controlled through the Permit to Work process.

Key Words

Confined Space Entry, CSE

HSE0085- Confined Space Entry and Work

- Purpose / Scope

- Audience / Roles and Responsibilities

- Confined Space: Identification and Classification

- Confined Space: Entry Requirements

  - Entry Actions and Documentation Requirements

  - Atmospheric Conditions

  - Atmospheric Testing

  - Personnel Safety

    - Ventilation Plan

    - PPE

    - Communications

  - Confined Space Preparation Considerations

    - Entry into Sour Service

    - Entry into Non-Sour Service

    - Welding and Burning

    - Pressurized Welding Enclosures (PWEs)

    - NORM

    - Isolation Requirements

- Confined Space Reclassification Process

- Confined Space Rescue Requirements

- Cleaning of Storage Tanks

- Confined Space: Alternative Methods of Safe Entry / Activity

Guideline Documents

- GL01- FPSO Stones Confined Space Supplemental Procedure

Tools:

- CS Authorized Attendant Form

- Gas Test Form

- Personnel Login/ Logout Form

- Reclassification Form

- CS Rescue Plan

- Start Work Checklist

# PURPOSE/SCOPE

This Procedure provides the guidance necessary to safely plan and enter a confined space. It supplements governmental/global CSE regulations and is to be used whenever a confined space entry is required on a DW-GOA facility unless covered by bridging document or other official procedural governing document which covers and instructs equivalently to this procedure.

Confined spaces are dangerous not only because of the physical hazards present, but also because the atmosphere can become contaminated due to leaks from connecting pipework or fumes from residues.

Because the space is enclosed, contaminated air is not refreshed, and the toxic components can build up to dangerous levels that can overcome workers, causing loss of consciousness and death. Belowground cellars and pits, even if their tops are open, should be treated as confined spaces, as they can accumulate gases heavier than air which will not naturally aspirate, accumulating at lower lying areas, thus presenting a safety hazard.

Health hazards can be addressed by a fit for purpose Health Risk Assessment (HRA).

Working in confined spaces has all the normal task risks, plus those due to limited access and egress alternatives. Recognition of these risks and how they will be addressed should be written into location procedures and/or the Permit to Work for the activity. CSW is controlled by the Permit to Work process.

| Training / Roles and Responsibilities |  |  |
| --- | --- | --- |
| ROLE; Rescue Team; CSE Competent Person; CSE Entrant; CSE Authorized Attendant; Qualified Tester. COURSE; Summary. Permit Vision Level 1 01010506; Training on the electronic Permit to Work system in UPD GoA; X; X; X; X. Permit Vision Level 2 01010515; Training on the electronic Permit to Work system in UPD GoA This course covers additional functions applicable to Verifiers/ Authorizers and specialized roles within the system. Rescue Training1 00115826; Industrial Rescue: PPE use, SDS, retrieval systems, SCBA, atmospheric testing equipment, packaging/retrieval of victims, rescue determination based on conditions, Rescue Plan development, and elevated high-angle rescue. Training required every 2 years; X. First Responder 01230602; On successful completion of this learning activity, participant will be able to take a patient's vital signs, take measures to stabilize a patient, assess patient injury, converse with a physician, nurse, or EMT and accurately describe a patient's condition/injuries, follow instructions of a physician or EMT for patient care, stabilization, and transportation, make recommendations for rescue or transportation of an injured person and apply CPR.; X. CSE for Entrant, Attendant 00929253; Covers definition of permit required confined space, hazards associated with each designation, atmospheric testing, etc.; X; X; X; X. CSE Competent Person 894510; Covers Shell/OSHA definitions/ regulations, PTW completeness, rescue plan overview, atmospheric testing, overall CSE plan preparedness, document review, and CSE crew qualifications (training, experience, etc.). Training required every 3 years; X. HSE Super Safety Course 1293812; Learning Safe Behaviors; General Safety, Personal Protective Equipment, Hazard -Communication, , Job Safety/Hazard Analysis & Job Safety Environmental Analysis (JSA/JHA/JSEA), etc.. Training required prior to entry or every year unless specified; X; X. MSA AltAir Gas Detector Training; https://www.youtube.com/@msanorthamerica424/search?query=AltAir%205 If detector is used; X. Draeger X-Am 2500 training; https://www.youtube.com/@draeger/search?query=xam%202500 If detector is used; X. Draeger X-Am 5500 Training; https://www.youtube.com/@draeger/search?query=xam%205500 If detector is used; X. Draeger X-Am 5600 Training; https://www.youtube.com/@draeger/search?query=xam%205600 If detector is used; X. Training; Knowledge; Skilled. Confined Space Supervisor / Competent Person. CSE Supervisor/ Competent Person 894510; Regulations, standards, equipment and systems, requirements for CSW. Workplace procedures, selection, maintenance and inspection of equipment, and hazards associated with confined space work. Supervision techniques, implementation and monitoring of projects requiring confined space work. Rescue techniques from confined spaces.; Identifies confined spaces. Supervises gas freeing and ventilation processes. Identifies the hazards associated with entering a confined space. Defines the required precautions with entering a confined space. Assists in confined space entry rescues (if trained and identified on Rescue Plan). Intervenes where others do not comply with the requirements and precautions associated with the entry of a confined space.. Confined Space Entrant. CSE for Entrant, Attendant 00929253; Have a working understanding of policies and procedures regarding confined space work, hazard identification, monitoring equipment use, respiratory protection donning and use. Checking equipment and confined space work environment before work commences and reporting anomalies.; Identifies the hazards associated with entering a confined space. Uses breathing apparatus in line with local requirements. Observes others whilst working in a confined space. Intervenes where others do not comply with the requirements and precautions associated with the entry of a confined space.. Confined Space Attendant. CSE for Entrant, Attendant 00929253; Have a working understanding of policies and procedures regarding confined space work, hazard identification, monitoring equipment use, respiratory protection donning and use. Knowledge of emergency response.; Identifies the hazards associated with entering a confined space. Uses breathing apparatus in line with local requirements. Observes others whilst working in a confined space.. Authorised Person for Gas Testing. Basic training in gas monitoring procedures and techniques. Training in the use and operation of applicable monitoring equipment.; Have a working understanding of policies, procedures, and techniques of gas monitoring, including interpretation of results.; Performs gas freeing and ventilation processes. Uses breathing apparatus in line with local requirements (if needed for gas testing).. Authorized Person for Respiratory Protection. Training in the selection and use of respiratory protection Training in facemask fit testing; Have a working understanding of policies, procedures, and techniques of respiratory protection.; Selects breathing apparatus and evaluates respiratory protection gear (including oxygen supply) in line with local requirements. Explain the requirements for the use of breathing apparatus.. Roles and Responsibilities: Qualified Gas Tester; Must be trained on the gas detector using for atmospheric testing Must be able to read, understand, and articulate atmospheric readings from gas detector(s) Must know the atmospheric conditions of the space prior to, during, and before reentry of the space Must document initial atmospheric readings as well as after every 30 mins of space occupied on gas test form Alarm entrants of any hazardous atmospheric readings and initiate evacuate if need arises Can be combined with Attendant, Supervisor, or Competent Person (not Entrant) as long as other duties can still be fulfilled Continuously monitor the atmospheric conditions of the space the entire time an entrant is inside the space.. CSE Authorized Attendant; Attendants shall be trained to perform the following duties: Be able to communicate with the entrants while inside the confined space in the event of an emergency. Be able to provide standby assistance to entrants entering the confined space. Direct entrants to exit the confined space when any irregularities are observed. Initiates evacuation and emergency procedures. Monitor for any conditions or changes that could adversely affect the entry. Monitor entry point and maintain communication with the entrant(s) Prevent unauthorized entry. Know the hazards within the space and activity Know the behavioral effects of the hazards. Be able to identify the authorized entrants. Remain outside until relieved. NOTE: Do Not Fall victim to the human impulse to rescue an identifiable person facing imminent threat, regardless of associated cost or risk. An attendant is NOT part of the rescue unless non-entry rescue is used and the attendant can still fulfill duties. Communicate with entrants throughout the work period. Monitor and evacuate entrants if necessary. Summon rescue, if needed. They should act on any observed changes, such as evidence of: - influxes of gases, liquids or solids; - equipment stopping working – particularly lights or ventilation; - inadequate supply or sudden pressure loss of supplied breathing air; - changes in atmospheric conditions inside the space; - people in physical difficulty, for example losing consciousness or struggling to work; - loss of visual or audible communication. In addition, the attendant may not undertake any additional duty that might interfere with these primary safety-related duties.. CSE Entrant; Know the hazards. Verify safe entry conditions. Terminate entry and cancel permit, initiate evacuation if conditions elevate. Verify availability and effectiveness of rescue services. Remove unauthorized persons. Ensure acceptable entry conditions are maintained. Communicate to Attendant if/when conditions inside space change. Identify changes in space and changes in risk profile/ mitigation application. Review work packet and sign, ensure all work party members are aware of the scope and risks.. CS Supervisor/ Competent Person; Know the hazards. Verify safe entry conditions. Terminate entry and cancel permit. Verify availability and effectiveness of rescue services. Remove unauthorized persons. Ensure acceptable entry conditions are maintained. Is familiar with and understands the hazards that may be faced during entry, including information on the mode, signs or symptoms, and consequences of the exposure. Verifies, by checking that the appropriate entries have been made on the permit, that all tests specified by the permit have been conducted and that all procedures and equipment specified by the permit are in place before endorsing the permit and allowing entry to begin. Terminates the entry and cancels or suspends the permit as required. Verifies that rescue services are available and that the means for summoning them are operable, and that the employer will be notified as soon as the services become unavailable. Removes unauthorized individuals who enter or who attempt to enter the permit space during entry operations. Determines, whenever responsibility for a permit space entry operation is transferred, and at intervals dictated by the hazards and operations performed within the space, that entry operations remain consistent with terms of the entry permit and that acceptable entry conditions are maintained. Ensure quality breathing air from supplied breathing apparatus.. CS Rescue Team; - An employer who designates rescue and emergency services must: Evaluate a prospective rescuer's ability to respond to a rescue summons in a timely manner, considering the hazard(s) identified; Note to paragraph (a)(1). What will be considered timely will vary according to the specific hazards involved in each entry. For example, § 1926.103 (Respiratory protection) requires that employers provide a standby person or persons capable of immediate action to rescue employee(s) wearing respiratory protection while in work areas defined as IDLH atmospheres. Evaluate a prospective rescue service's ability, in terms of proficiency with rescue-related tasks and equipment, to function appropriately while rescuing entrants from the particular permit space or types of permit spaces identified; Select a rescue team or service from those evaluated that: - Has the capability to reach the victim(s) within a time frame that is appropriate for the permit space hazard(s) identified; - Is equipped for, and proficient in, performing the needed rescue services; - Agrees to notify the employer immediately in the event that the rescue service becomes unavailable; Inform each rescue team or service of the hazards they may confront when called on to perform rescue at the site; and Provide the rescue team or service selected with access to all permit spaces from which rescue may be necessary so that the rescue team or service can develop appropriate rescue plans and practice rescue operations. An employer whose employees have been designated to provide permit space rescue and/or emergency services must take the following measures and provide all equipment and training at no cost to those employees: - Provide each affected employee with the personal protective equipment (PPE) needed to conduct permit space rescues safely and train each affected employee so the employee is proficient in the use of that PPE; - Train each affected employee to perform assigned rescue duties. The employer must ensure that such employees successfully complete the training required and establish proficiency as authorized entrants - Train each affected employee in basic first aid and cardiopulmonary resuscitation (CPR). The employer must ensure that at least one member of the rescue team or service holding a current certification in basic first aid and CPR is available; and - Ensure that affected employees practice making permit space rescues before attempting an actual rescue, and at least once every 12 months, by means of simulated rescue operations in which they remove dummies, manikins, or actual persons from the actual permit spaces or from representative permit spaces, except practice rescue is not required where the affected employees properly performed a rescue operation during the last 12 months in the same permit space the authorized entrant will enter, or in a similar permit space. Representative permit spaces must, with respect to opening size, configuration, and accessibility, simulate the types of permit spaces from which rescue is to be performed. Non-entry rescue is recommended unless the retrieval equipment would increase the overall risk of entry or would not contribute to the rescue of the entrant. The employer must designate an entry rescue service whenever non-entry rescue is not selected. Whenever non-entry rescue is selected, the entry employer must ensure that retrieval systems or methods are used whenever an authorized entrant enters a permit space, and must confirm, prior to entry, that emergency assistance would be available in the event that non-entry rescue fails. Retrieval systems must meet the following requirements: Equipment that is unsuitable for retrieval must not be used, including, but not limited to, retrieval lines that have a reasonable probability of becoming entangled with the retrieval lines used by other authorized entrants, or retrieval lines that will not work due to the internal configuration of the permit space. If an injured entrant is exposed to a substance for which a Safety Data Sheet (SDS) or other similar written information is required to be kept at the worksite, that SDS or written information must be made available to the medical facility treating the exposed entrant.. |
| Confined Space: Identification and Classification |  |
| Step 1: Identification: | "Confined space" is defined as a space that: - Is large enough and so configured that an employee can bodily enter and perform assigned work; and - Has limited or restricted means for entry or exit, and - Is not designed for continuous employee occupancy. CSE shall only be allowed in the following cases: No other way can be found to perform necessary work. It is required to meet a government regulation (BSSE, OSHA, USCG, State, or local) or a DW-GOA requirement. The space has been: - secured, cleaned, and ventilated, - atmospherically tested, and - declared free of any materials that could be harmful to entrants Such spaces must be labeled as Confined Spaces (respective to their level), with signs posted conspicuously (per 29 CFR 1910.146(c)(2)). OSHA does not, however, require the posting of any permit space where tools or keys are needed to gain access, as long as personnel expected to gain access are trained per Letter of Interpretation dated 9/21/2006. Confined spaces with hatches in the side as well as the top shall be entered from the side, whenever possible, to facilitate: adequate Entry Team access to the jobsite within the confined space, a free and clear path for removing incapacitated Entry Team members, and sufficient work space to conduct safe and efficient operations. When confined space work requires platforms and/or access ladders, use HSE0044 Fall Prevention and Protection, and barricade the area to prevent unauthorized access. Enclosed work environments/ enclosed spaces shall be evaluated to determine if they are Level 1, 2, or 3 confined spaces, based on the definitions below. IMO resolution A.1050 (27) - enclosed space means a space which has any of the following characteristics: - Limited openings for entry - Inadequate ventilation; and - Not designed for continuous worker occupancy, and includes, but is not limited to, cargo spaces, double bottoms, fuel tanks, ballast tanks, cargo pump-rooms, cargo compressor rooms, cofferdams, chain lockers, void spaces, duct keels, inter-barrier spaces, boilers, engine scavenge air receivers, sewage tanks, and adjacent connected spaces*. *Adjacent connected spaces are normally unventilated spaces not used for cargo, but which may share the same atmospheric characteristics with the enclosed space, including, but not limited to, a cargo space access way. Confined Space Signage: Entry into and exit from the confined space must be controlled and recorded by the Attendant. If there is more than one entry point they must all be monitored. Entry points on the sides of vessels and tanks are preferred to those on the top, as it is easier to come in and out. Entry points should be as large as possible and should be kept clear both outside and inside. All other openings to the confined space are non-entry points and should be barricaded and signed as non-entry points. These may be required for ventilation, so they should not be sealed. Consider scheduling Confined Space Entries (CSE) during winter months to avoid heat related illnesses. Confined Space Entry (CSE) pre-job planning should include focused hydration efforts by entrants the day prior to entry. Acclimatization for entrants should be considered prior to any Confined Space Entry (CSE) work. Delay entries where necessary for adequate adaption to environment. Receive verbal confirmations by entrants of fit-for-task readiness during Toolbox talk prior to each Confined Space Entry (CSE). Consider documenting confirmation on wet inked JSA’s. |
| Step 2: Classification: | Classification; Definition. Level 1 (L-1); Does not contain/have potential to contain hazards capable of causing death or serious physical harm. Includes permanent reclassified Level 3 spaces (crane pedestals, active hull columns, ballast/void tanks, vessels, etc.) Permanently reclassified (see Reclassification to L-1 below).. Level 2 (L-2); Level 3 (L-3) confined space with no flammable/toxic liquids/gases, with all hazards eliminated without entry, and no longer meeting L-3 definition below. Short-term duration (< 60 days) temporary reclassified L-3 spaces, which will be considered L-3 until deemed hazard-free by CSE Competent Person. If space must be entered to eliminate hazards, CSE shall be performed under L-3 requirements (1910.146(c)(7)(ii).. Level 3 (L-3); Contains/might contain atmosphere not aligning with atmospheric conditions below (Atmospheric Testing), Contains material that can engulf an entrant. Structured such that entrant might be trapped/asphyxiated (walls converge inward, floor slopes down and tapers to smaller cross-section, etc.). Contains/might contain hazards capable of causing injury or acute illness and impair an entrant’s ability to self-rescue.. |

| Confined Space: Entry Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required Risk Mitigation and Documentation; CSE Level. L-1; L-2; L-3. Permit for work (e.g. Cold Work, Hot Work, etc.) NOTE: For L-2, Permit shall cover preparations, entry and work on permit.; X*; X; X. CSE permit for preparations and entry (shall remain live until work is complete and space is ready to be put back in service.) *L-2 permit is allowed to cover entry and work.; X; X. Risk Assessment/Job Safety Analysis specific for the Confined Space work / entry; X; X; X. Gas Detector and Gas Test form included in permit packet NOTE: MSA Alt Air 5 detector or Draeger X-Am 2500 (with attached X-Am pump)or X-Am 5500 are required for initial, preferred for subsequent. Ensure training on gas detector is completed before operating.; X; X; X. Atmospheric Testing and a Hazard Assessment of space (inside and out) before entering. NOTE: For L-2 CSE, entry to eliminate hazards must meet L-3 CSE requirements per 1910.146(c)(7)(ii). NOTE: For L-3 CSE, atmospheric testing is conducted just before entry, in the presence of the OIM/PIC to suffice the requirement of Start Work checklist.; X; X; X. Completed Reclassification CSE form.; X; X. Safe Work Plan covering specific information about the entry and work to be completed (activity/ campaign level as necessary dependent on risk); X. Rescue Plan completed and Team Briefed about entry; X; X. Rescue Team gear set up and ready at confined space entrance.; X; X. Attendant or use of buddy system while inside space; X; X; X. Personnel Log In/Log Out form (maintained by Attendant); X; X. Ventilation (as space atmosphere or weather conditions necessitate); X. Lock Out Tag Out (LOTO) with blinds (unless risk assessment is completed identifying additional/alternative controls to ALARP); X; X. Holewatch Form maintained by Attendant; X. Confined Space Start Work Checklist (OIM/PIC signs after witnessing atmospheric test and before entry); X; X. Local Operating/written procedure (if available); X; X; X. *= As needed, dependent on activity being conducted inside L1 space. Stop work if hazards are detected within space. If hazards cannot be eliminated for L-1 and L-2 CSE, close permits and reissue as L-3 CSE PERMIT (1910.146(c)(6)). Follow Level 3 CSE requirements.. |
| Documentation Requirements |  | Level 1: |  | Control of Work Document for task/ activity - Activity may require multiple documents dependent on scope Risk Assessment/ JSA for task /activity Reclassification Form (completed for that space and filed on site) Gas Test Form (for initial gas test reading logging) Written instruction or equivalent |
|  |  | Level 2: |  | All of the above, plus: Permit covering entry preparations and activity to be completed within space Rescue Plan completed and presented to Rescue Team - Rescue Team briefed of entry and space configuration, controls, etc. Ventilation Plan (can be included in Control fo Work/ Permit to Work documents LOTO with blinds unless properly risk assessed* Personnel Login/ Logout Form Confined Space Start Work Checklist |
|  |  | Level 3: |  | - All of the above, plus: Separate Permit for Entry/Preparations and one for scope of work to be completed inside space Safe Work Plan Holewatch Form Minus Reclassification Form |
| Atmospheric Conditions |  | Before entry, the space shall be tested by a Competent Person or Atmospheric Tester, using a calibrated direct-reading instrument for (in this order): oxygen content, flammable gases and vapors, and potential toxic air contaminants For multi-tiered confined spaces, the atmosphere in each space must be tested and the procedure followed for each tank or vessel. Entry requirements depend on atmospheric limits per the table below.. Atmospheric Component; Entry without Supplied Breathing Air; Entry with Supplied Breathing Air*; Entry Not Allowed. O2; 20% to 21.5%; > 16% to 20%; ≤ 16% or > 21.5%. Toxins; 50% of PEL**; < IDLH***; ≥ IDLH. Flammables; < 1% LEL****; < 10% LEL; ≥ 10% LEL. Dust; > 5' visibility; > 5' visibility; ≤ 5' visibility. Temperature; < 120° F; < 120° F; < 120° F. * Entry with supplied breathing air is only allowed if more favorable conditions cannot practically be achieved. Such entry shall only be for cleaning, rescue and/or inspection. ** The PEL (Permissible Exposure Limit) (or OSHA OEL) is the maximum amount or airborne concentration of a substance to which a worker may be legally exposed. *** Immediate Danger to Life or Health (IDLH) applies to an atmosphere posing an immediate or delayed threat to life or health. **** If gas detector reads > 0 LEL, stop the job. Alert immediate supervisor and Area Owner. Find and eliminate the cause of the LEL if safe to do so, and document the actions taken on the JSA. Only restart job after favorable atmospheric conditions are maintained. |
| Atmospheric Testing |  | Before entry, open as many hatches as possible to provide natural ventilation. NOTE: When entering void/ballast tanks, consider potential for hazardous atmospheres (H2S due to rotting organic materials; oxygen depletion through corrosion, NORM, etc.). Before entry, enough air must be injected to exchange at least 3 times the volume of the confined space. This purge may take more than 24 hours for a vessel that previously contained hydrocarbons. Forced air used to ventilate a confined space must be free of contaminants. Route exhaust to a safe location. WARNING: Never use pure oxygen to correct oxygen deficiency in a confined space. Before the permit is issued and work commences, an Authorized Gas Tester must monitor the atmosphere in the confined space using a suitable instrument which measures oxygen level, % LEL and any toxics which could be present. When the results confirm levels are again within the limits described above, the permit can be issued, and the work can begin. The results of the gas test must be recorded and attached to the permit. Initial testing is best done at the ventilation outlets. Other areas of the confined space can be sampled using an extension tube on the gas monitor, or by lowering the gas monitor into the space and taking readings at certain positions. If required as confirmation of safe conditions, the Authorized Gas Tester should enter the confined space with respiratory protection for final testing. Heavy gases and vapors may accumulate at low points, so these should be carefully checked. If the initial gas testing shows any levels are outside the set limits, the permit cannot be issued, and work cannot commence. Further work needs to be performed to investigate the source of the hazardous atmosphere, for example if an isolation is missing or passing, or if a residue has not been properly removed. If at any time testing shows the atmosphere in the confined space to be outside set limits, the occupants should evacuate. Work should be suspended, the issue investigated, and the source determined and resolved. Work can only be restarted when monitoring shows that the atmosphere is stable and back within the set limits Only once the source is remedied and measurements show the atmosphere meets the levels set can the permit be issued and work begin. Provide continuous forced air ventilation until work within the space is complete. Job Supervisor shall assign competent personnel to monitor blower operations and ventilation velocities. Externally ground ventilating fans using green wire per table below and lugs or bolts (no alligator clips). After ventilation, sample hull/void spaces at multiple levels from top to bottom using a 50' or shorter extension hose attached to oxygen and combustible gas meters. For depths more than 50', use a rope or tubular to lower detectors to the bottom of the space. Pay particular attention to low areas such as sumps for heavier-than-air gases, and restricted areas at the top of the space for lighter-than-air gases Take Note: The sampling read-out may be delayed some seconds due to the length of the attached sampling hose (general rule of thumb: 1ft of hose= 1 sec delay in reading) Personal / Continuous monitoring is required while entrant is inside space and when any atmospheric hazard exists. Personal / Continuous monitoring is allowed using personal gas detectors as long as it is multi-gas, has an audible alarm, and calibrated/ bump tested before use (in accordance with manufacturer’s recommendations) Atmospheric testing shall be performed, and readings documented: Before entry Every 30 minutes thereafter When conditions (e.g. temperature) change When leaving the space for extended period of time (+30 mins) After work breaks, before re-entry |  |  |
| Test Equipment Requirements |  | Measurements should be taken by the Authorized Gas Tester to confirm that the atmospheric limits described above are being met, both before entry and during the work. There are three generic types of portable gas detectors: - Personal detectors are small matchbox-sized instruments worn on the clothing, which should be attached to monitor the breathing air of the wearer. - Hand-held instruments are larger, carried in a harness, and portable to test at various locations. - Area monitors are larger still, freestanding, and monitor a specific location while work is carried out. Most modern hand-held and area monitors are multi-gas detectors: the same instrument can measure: oxygen depletion, flammability, and toxicity (for example hydrogen sulphide, carbon monoxide and/or carbon dioxide). The authorized gas tester is responsible for understanding the hazards to be evaluated in the gas space, and for using the appropriate atmospheric testing equipment. Test equipment shall comply with OSHA 1910.146(c)(5)(ii)(C) and this Procedure. It shall also meet Class 1, Division 1 electrical requirements. “Alarm only” devices that do not provide readings are not acceptable. Never use an instrument that cannot be calibrated or fails a bump test. Combination units shall include a pump, a meter or display showing actual concentrations, and a pre-set alarm providing real time concentration information and automatic (unattended) alarming at a pre-determined value. Gas detectors must: be pump fed for continuous monitoring and described as such in the manufacturer’s specifications. detect oxygen content, flammable/combustible gases and vapors, and potential toxic air contaminants (in that order) in specific concentrations and must relay hazardous conditions to the operator. have the attachments needed to test atmosphere before entering (e.g. sampling hose, additional pump if needed). MSA Altair 5/5x portable multi-gas detectors and the Draeger X-Am 2500 (with accessory pump required) or Draeger X-Am 5500/5600 are the required units for initial (daily) CSE testing. NOTE: Shell has renewed EFA agreements with both Draeger and MSA in 2021. If you need to order, I recommend contacting your C&P rep to determine best way to order to take advantage of competitive EFA pricing. |  |  |
| Test Equipment Calibration and Bump-Testing |  | Gas detectors must conform to the requirements above and be: bump-tested once per shift while in use (using known concentration of [calibration] gas [i.e. Pentex], and must respond within +/-10% of the calibration gas concentration) and calibrated per manufacturer’s instructions, or at least every 30 days. Never use an instrument that cannot be calibrated or fails a bump test. Send these to the manufacturer for maintenance or repair. Calibration and bump test result records must be kept retrievable for inspection/review in paper (e.g. log book) or electronic format, under the DW-GOA Records Retention Policy (see HSE0138-PR01). Automated calibration docking stations should be used to meet these requirements: MSA Galaxy Automated Test Systems must be used to calibrate, test and log the Altair 5/5x. The Draeger detectors are to use the X-Dock system for calibration and bump testing logging |  |  |
| Personnel Safety |  | Ventilation Plan | If contaminants or heat in the Confined Space can affect entrants’ health, provide a plan for ventilation or other Controls prior to Entry, list the Controls with the ventilation plan and verify that the Controls are put in place. For instance, engineering calculations should be performed to assess accumulation of contaminants and/or depletion of oxygen, and what steps will be required to maintain the atmosphere within the levels. For example, exhaust ventilation should have the capacity to remove contaminants at the required rate, and mechanical ventilation should be able to provide the necessary air changes. These requirements should be placed on the permit, together with the controls to ensure they are not interrupted. Natural ventilation should be used where possible, both in the preparation of the confined space and during the work. Hatches and other openings should be left open where possible. Mechanical ventilation is a recognized method of providing clean and cool air to a confined space. This can either be: exhaust ventilation, which sucks contaminated air out of the confined space; or forced air ventilation, which blows clean air into the confined space. It should be used both in the preparation of the confined space and during the work. Before allowing work to start, the supervisor should confirm that the controls listed on the permit are in place and working properly. Before entry, open as many hatches as possible to provide natural ventilation. NOTE: When entering void/ballast tanks, consider potential for hazardous atmospheres (H2S due to rotting organic materials; oxygen depletion through corrosion, NORM, etc.). Before entry, enough air must be injected to exchange at least 3 times the volume of the confined space. This purge may take more than 24 hours for a vessel that previously contained hydrocarbons. Forced air used to ventilate a confined space must be free of contaminants. Route exhaust to a safe location. WARNING: Never use pure oxygen to correct oxygen deficiency in a confined space. Provide continuous forced air ventilation until work within the space is complete. Performing Technician shall assign competent personnel to monitor blower operations and ventilation velocities. Externally ground ventilating fans using green wire per table below and lugs or bolts (no alligator clips). Below is a chart depicting ground wire specs for air movers/blowers: Ventilation Fan Ground Wire Specifications. Amperage; Gauge. Up to 160 A (FLA); #6 AWG. 161 A (FLA) to 400 A (FLA); #2 AWG. 401 A (FLA) to 800 A (FLA); #2/0 AWG. 801 A (FLA) to 1280 A (FLA); #4/0 AWG. Above 1281 A (FLA); see NEC Table 250.122. The ventilation plan must be covered in the Control of Work documents for the activity(ies) and must specify the following: Ventilation Type Ventilation equipment to be used Configuration (to include ductwork if used) Location(s) and timing Energy Tie-Ins Set Up/ Prep actions |
|  |  | PPE | PPE is the last barrier to people being exposed to a hazardous condition. An Asset or Business should follow ANSI/ASSP Z117.1-2022 Safety Requirements for Entering Confined Spaces Section 10.1 “Confined spaces shall be cleaned and decontaminated of hazardous materials as the preferred method of eliminating or reducing exposure. Cleaning and decontamination shall be done to the extent feasible before entry. Note: In some instances, the purpose of the entry is to clean the confined space. In these cases, the confined space should be cleaned and decontaminated as much as possible. before personnel enter. Proper PPE and other precautions should be used to address any hazards, which will remain after the pre-entry cleaning. Prior to commencing cleaning and decontamination, care should be exercised in the selection of cleaning compounds to ensure their compatibility with the environment in which they will be used.” |  |
|  |  |  | PPE Type; Requirements. Gloves; Must protect the hands and be designed for the task.. Disposable Protective Clothing; Must protect entrant’s clothing and skin from residue within the space.. Respiratory Protection; Must be selected per HSE0005 Respiratory Protection and reviewed by a Competent Person. Respiratory Protective Equipment (RPE) is specifically designed to ensure that the air being breathed through it is not toxic or harmful. This is done in one of two ways: using a facemask to filter or absorb contaminants from the external air; or delivering a supply of clean air through an airline to a facemask. The air supply can be from an external point or from a portable air tank. The quality of the supplied air needs to be assured. The selection of RPE must be done by an authorized, competent person. It requires knowledge of the level of each contaminant in the air and the safe breathing levels to calculate the Nominal Protection Factor (NPF). The NPF determines the type and model of respiratory protection to be used.. Hearing Protection; Must be provided per HSE0007 Hearing Conservation Program if continuous forced air ventilation is used.. Safety Harnesses/ Retrieval Gear; Approved safety harness that suspends wearer in an upright position, 600 lb test lifeline, and industry approved rescue retrieval device are required for entry through top hatch (see HSE0044-PR01 Fall Prevention and Protection). Per OSHA 1910.146(k)(3), if CSE Competent Person and OIM/PIC/Drilling Supervisor determine that retrieval equipment would increase risk of entry or would not contribute to rescue, an exception can be made.. Rescue Team Support Equipment; Ready-to-use SCBAs shall be available at the jobsite. Gear shall be inspected before use and be in good working condition. Hose from manifold to regulator must be supplied by the manufacturer and stamped with the manufacturer’s name. It must not exceed 300' in length. Preferred hose material from source to manifold is Synflex. There is no minimum or maximum length required for this hose.. |
|  |  | Communications | Equipment, including backup, needed to maintain communications for: conducting entry and executing rescue, offsite medical personnel, Rescue Team, and Control room (if applicable). Must be selected prior to commencement Checked before use Means for secondary communication (if primary fails) Visual and/or audible contact shall be maintained between the Entry Team and the Authorized Attendant at all times. If direct visual contact is not possible, approved audio communication equipment shall be used. A way to signal either party that immediate evacuation is needed shall be established in advance. |  |
| Space Preparation Considerations |  |  |  |  |
| Entry into Sour Service |  | When hydrogen sulfide or other toxic material is involved, completely purge equipment before inspection for CSE to make sure that ventilated toxic material is disposed of safely. Special consideration shall be taken in cases where Pyrophoric Iron Sulfide can build up in the space. Pyrophoric iron sulfide is a product of corrosion on the walls of vessels/tanks containing high sulfur or sour stocks. In the presence of air, it can spontaneously combust. Pyrophoric iron sulfide ignition can be prevented by isolating the deposits from air or wetting interior equipment surfaces of the equipment with water using fog nozzles, water sprays, or condensing steam. The steps below are recommended for vapor-freeing and removing pyrophoric deposits from tank shells and roofs Step; Action. 1; Steam tank until interior surfaces are wet with condensate.. 2; Install a high-capacity air mover in the roof manhole and a large fog nozzle in the neck of the shell manhole. CAUTION: Electrically bond air mover and fog nozzle to the tank to prevent static electricity buildup.. 3; Turn on the fog nozzle, then immediately turn on the air mover. Continue running both until the tank is vapor free.. 4; With the air mover still running, remove the fog nozzle, then knock down loose scale with a high-pressure water stream. NOTE: Workers entering the tank with high-pressure water line must wear adequate protective equipment.. 5; Proceed with the tank cleaning operation.. |
| Entry into Non-Sour Service |  | Steaming Service | Confirm vessel insulation/protective coating can withstand steam temperature. Provide sufficient openings to prevent pressure build up inside the vessel. Steam should be injected near the center of the shell in vessels without trays and below the bottom tray in vessels with trays. Vessels like separators or long exchangers are best steamed by injecting near the center and venting from both ends and the bottom. Auxiliary equipment (gauge glass columns, level control bridles, etc.) should be vented. Remove temperature-sensing devices that might be damaged/over-ranged. The bottom opening should be large enough to drain liquids. Provision should be made for safe disposal of this liquid, which will likely contain hydrocarbon. Be prepared for heat expansion of steamed equipment. Do not block-in and allow steam to cool in a vessel not designed for vacuum. Confirm equipment is hydrocarbon-free by reducing injected steam volume, ventilating the space, and checking the outer edge of vented steam with a combustible gas detector. - NOTE: Combustible gas detector cannot be used directly in steam, and this check is not conclusive and may damage the detector. Make the final combustible gas check on atmosphere inside the vessel. If hydrocarbons are detected, repeat steaming until vessel is hydrocarbon free. |  |
|  |  | Water Displacement | If steam is not readily available, or construction/piping configuration make steaming ineffective, equipment can be purged by water displacement. Usually, water is pumped into the bottom of equipment until it is completely filled, forcing gas and floating liquid hydrocarbon out the top. Equipment must be able to withstand the force of a full water column and vacuum during draining. Detailed instructions that apply in every case cannot be provided. OIM/PIC must use sound judgment in planning this job. |  |
|  |  | - Air Purging | Purge an L-2/L-3 space with at least 3 times the confined space’s volume of air before entry. Continue purging until equipment is gas free. Continue ventilating as long as workers are in the confined space. OIM/PIC shall decide on type of air mover, whether to purge from top or bottom, size and number of openings, and if weather poses undue hazards. OIM/PIC shall obtain concurrence from the Operations Manager before vapor freeing by air purging. Only air purge equipment containing vapors and no liquid hydrocarbons Do not air purge sour service equipment. It might have pyrophoric deposits. • High volume air movers are recommended to minimize the time a vessel/tank contains an explosive/combustible mixture. Forced or induced draft ventilation is preferred. If a blower driven by an electric motor is used, the motor must be explosion proof. Sparking devices are not permitted within 150' of equipment to be purged. Equipment should not be vented when lightning may cause vapor ignition. Limit personnel working in the immediate area during purging |  |
| Welding and Burning |  | Welding inside equipment must be minimized, and qualified DW-GOA personnel and CSE Competent Person shall confirm that the space is safe for this work. Use local exhaust ventilation (see OPS0096A Welding/Burning). Welding is not allowed within a firewall or within 150' of equipment if it can be moved to a safe area. Welding is not allowed on a leaking bolted tank bottom unless the leaking section is moved outside the firewall at least 150' from the tank. Repairs on welded vessels require that the interior atmosphere be declared safe and the work area (oily skidpan, etc.) be prepared to prevent ignition. Handle flame cutting components (pressurized cylinders, gas hoses, valves, etc.) with care and inspect daily for damage. Replace damaged components. Remove flame cutting/welding equipment (welding leads, torch heads, hoses, etc.) from confined spaces during breaks and at end of shift. |  |  |
| Pressurized Welding Enclosures (PWEs) |  | Welding within a 10' radius of a wellhead, wellbay, or production area can only take place in a PWE unless a departure is obtained from the BSEE. PWE’s must be: approved by the appropriate BSEE district office durable and constructed of materials compatible with welding/burning Work inside a PWE requires a Safe Work Plan which must contain: Detailed description of the work to be performed Start-up and shut-down procedures Requirements for work-stoppage and evacuation Work in a PWE is a special case but is not considered a “confined space”. See OPS-0096 for more details and instructions on the use of PWEs. PWE’s contain robust safety detection and shut down systems. With OIM/PIC approval, venting and bleeding of hydrocarbons is allowed while a PWE is in service. Discussions should consider proximity of air intake and location of PWE in relation to the area where venting/bleeding activities will occur. Please refer to the Shell Manual of Permitted Operations (MOPO) for additional guidance. |  |  |
| NORM |  | Use caution during hot work on vessels with possible NORM buildup. (Welding vaporizes NORM scale, which cannot be removed from the body if inhaled.) NORM scale must be removed before welding is permitted. Check vessels that contained produced water with radioactive sensing equipment (Ludlum Model 3 with 44-2 probe). Check entrants leaving a NORM contaminated vessel checked with an Alpha wave radioactive meter (Ludlum with Alpha paddle). Remove contaminated clothing before leaving the barricaded area. See DW-GOA NORM Website for more information. |  |  |
| Electrical Equipment/ Lighting Conditions |  | If electrical equipment is required for use in Confined Space, low voltage tools, double insulated or earth current leakage devices (e.g. residual current device, ground fault circuit interrupter) shall be used. A risk assessment should be carried on any equipment by identifying the hazards, assessing the risks, and using the Hierarchy of Controls to select the control and recovery measures. In wet or dirty environments, there is a risk that electrical cables will get damaged, particularly if the cables have to pass through the entry point and small openings. In this case, it is best to use air-driven equipment if possible. Once the controls have been selected, they should be captured in the work plan and the Permit to Work. The lighting level in the confined space needs to be at a minimum of 5 lumens for the work to be conducted and allow for safe egress in the event of an emergency. To prevent electrical shock, it is recommended that this requirement follow the Hierarchy of Controls; use low voltage equipment when inside a confined space. If it is not possible to use low voltage equipment and there is potential for electrical shock, an asset or business should use the appropriate electrical equipment or systems. Consideration of the lighting conditions within the Confined Space shall be taken into account prior to entry into the space. If the lighting or visibility is degraded, artificial lighting shall be used to illuminate the space. Consider low-voltage lighting or intrinsically safe lighting to minimize risk of electrocution or electrical shock. For work planned in confined spaces, electric lamps, tools and equipment should be used as follows: - Compressed air driven or with their own internal power source (battery), - Operated at maximum 50V a.c. with an isolating transformer (sometimes referred to as Extra Low Voltage ELV), - Operated above 50V a.c. with double insulated tools and isolating transformer, - Directly connected to mains power using RCD / GFCI (per local requirements). If main (high) voltages are used, either an earth leakage current device or a ground fault circuit interrupter must be used to protect from electrical shock. These devices compare supply and return currents and shut down the supply when they detect a difference, which indicates a loss of current to earth. Equipment should be inspected before use and controls should be in place to prevent damage to the power supply cable. |  |  |
| Isolation Requirements |  | Shell Control Framework Mandatory Manual for Safe Isolation mandates that blinding/ positive isolation is the minimum standard for Confined Space. Due to the nature of operations in DWGoA, blinding may not be the most feasible or safe option. Each Confined Space shall be evaluated on a case-by-case basis, considering configuration, energy sources, scope of activity, etc. to determine the isolation needs of the space. If blinding/ positive isolation is NOT applied, then a risk assessment specific to the isolation scheme must be completed, to include additional controls and ancillary isolation points to suffice positive isolation integrity. Isolations should cover electrical pathways and mechanical equipment, pneumatic and hydraulic equipment as well as any other energy source present during the job. Preparation to enter includes assessment of operating service and how it can be emptied, flushed and cleaned, ideally to support life without additional PPE; this could include: flushing, detergent-cleaning and water-washing to remove harmful residues. purging followed by air ventilation (e.g. using blowers). steaming out (ensuring no negative heat impacts). use of water sprays and lances to address possible flammable atmosphere (e.g. prior to tank entry). The aim is to make the confined space clean and safe enough for people to work in, without the need to wear chemical suits, respiratory equipment, hearing protection, lifelines, which although maybe necessary to preserve life introduce other hazards. Confined spaces are usually small and cramped, so such equipment is at best a nuisance, and at worst a snagging risk. It also tends to isolate the person from what is going on around them, reduce their situational awareness and make escape more difficult. A particular hazard is ionizing radiation, both from Naturally Occurring Radioactive Material (NORM) and from radioactive sources attached to the vessel. Often vessels have level gauges attached which contain a radioactive source. These should be removed or shuttered shut while the space is occupied. |  |  |
| Confined Space Reclassification Process |  | Permanent L-1 CSEs must be approved annually by the CSE Competent Person, OIM/PIC/Drilling Superintendent, and Operations Manager (OM), and shall include how or why the space poses no actual or potential atmospheric hazards. Each authorization signature will be recorded using the Reclassification form and stored locally for ease of access. Permanent reclassified confined spaces examples may include but are not limited to crane pedestals, active hull columns, ballast/void tanks, non-active vessels, etc. The list of L-1 confined spaces and their approvals shall be maintained on-site in either written or electronic format. L-2 (temporary reclassification) is for a duration < 60 days and requires evaluation by CSE Competent Person. It includes how/why the space poses no actual or potential atmospheric hazards, with sign-off by location Person in Charge using CSE Reclassification Form. The checklist shall accompany the permit packet at the worksite for reference and justification. Any identification of hazards that are not eliminated prior to entry shall be eliminated before entry or the space must be classified as a L-3 to enter and mitigate until hazards are eliminated. Maintain a list of L-1 confined spaces and approvals on site, either written or electronic. Permanent L-1 CSEs must be approved annually by: CSE Competent Person OIM/PIC or Drilling Superintendent Operations Manager (OM). Approval shall include how or why the space poses no actual or potential atmospheric hazards. Record approval using form and store this form on site. To reclassify multi-tiered L-3 CSEs, the CSE Competent Person evaluates each confined space to deem it hazard free, starting with the first CSE and progressing through all spaces. An attendant shall be posted at each space entrance (e.g. for a ballast tank inside a hull column, there will be at least 2 attendants: 1 at the hull column entrance, and 1 at the ballast tank entrance). |  |  |
| Confined Space Rescue Requirements |  | Planning for Rescue | DW-GOA Rescue Team SHALL conduct annual CSE rescue drills to assess Plan under various conditions (suggest drills within L-1 space, covering likely scenarios). Consider response time and rescue capability based on site-specific conditions/hazards when using an external Rescue Team. |  |
|  |  | During a Rescue | Authorized Attendant’s first priority is to make notifications of the emergency. Authorized Attendant shall not enter confined space until help arrives at entrance. Rescue Team’s first priority is to establish if emergency is due to confined space atmosphere contamination. Rescue requires SCBA unless the atmosphere poses no threat to personnel. |  |
|  |  | Personnel | List of trained rescue team members, including relief. All must be familiar with configuration of equipment. Definitions of roles and responsibilities for: Rescue Team members Paramedics and first responders Medical first aid providers |  |
|  |  | Notification | Agreed procedures for notification (who/what/when/how) of: internal (DW-GO) emergency response personnel, and external parties (e.g. medical provider, USCG) |  |
|  |  | Rescue Equipment | Self-Contained Breathing Apparatus (SCBA) Harness Retrieval device (for vertical entry) Lifeline usage Supplies for resuscitation and first aid located at the jobsite Fire protection equipment Artificial lighting (see OPS0177A-SP01; intrinsically safe lighting preferred) |  |
|  |  | Communications Equipment | Equipment, including backup, needed to maintain communications for: conducting entry and executing rescue, offsite medical personnel, Rescue Team, and control room (if applicable). |  |
| Cleaning of Storage Tanks |  | Before cleaning of CS begins, discuss and document how the process will take place, resources needed for completion, certain components of the activity (personnel, PPE, documentation, gas testing, etc.) Use the documents below to assist in defining the plan: API 2015 Requirements Safe Entry and Cleaning of Petroleum Storage Tanks API 2016 Guidelines and Procedures for Entering and Cleaning Petroleum Storage Tanks Cleaning of Storage Tanks Mandatory Manual Before entry and activity commencement, discuss and decide on the best approach to the activity by assessing the Hierarchy of Controls: First: Minimize the need for or reduce the frequency of tank cleaning. Second: Use Online Cleaning methods that do not require the opening of, or Entry into, tanks. Third: Use mechanical cleaning options that do not require people to enter tanks. Fourth: Allow personnel entry and manual cleaning of tanks where supported by a documented Risk Assessment. Reference Confined Space Work. Refer to the local Tank Cleaning Procedure or The Cleaning of Storage Tanks Mandatory Manual for further instructions on safe tank cleaning. |  |  |
| Confined Space Alternative Methods of Safe Entry/Activity |  | Optimize your equipment design at first available opportunities to enable deployment of nonhuman entry solutions including process cleaning (e.g. suitable nozzles or manhole to allow robots to enter) and to eliminate the need for entry driven by repairs, e.g. of failing internals, or degrading coatings or refractory. The safest approach is to do as much of the work as possible outside the confined space. For example: Consider non-intrusive inspection solutions for pressure vessels, complemented with the use of remotely deployed camera & inspection tools, if internal surfaces must be inspected but can’t be accessed with non-intrusive inspection. Consider robotic/drones for internal inspection of i.e. storage tanks, furnaces, chimney stacks boilers, sump caissons, sludge catchers, catalyst reactors, trenches, etc, where appropriate, complemented with non-intrusive inspection techniques applied from the outside. If the deposit samples need to be taken, it can be collected without entry using remote sampling tools. If settled solids (such as sand or silt) have to be removed from separators, tanks or pipework, a pre-treatment with water or chemicals can often be used to flush the vessels clean, or at least remove the bulk of the material, before the vessels are opened. Jetting with a tank wash head or long lances from outside the confined space can also be used. Technology has advanced to allow CSW workers to reduce exposures to personnel while maintaining/increasing oversight and ensuring safe completion with less personnel required. One of these “alternative methods” of CSW oversight are: Centralizing CSW Monitoring This section will provide structure and foundational guidance on this option as what should be considered and included when selecting to use this option. Centralized CSW Monitoring refers to the use of technology (cameras, microphones, RFID (Radio Frequency Identification), etc.) in conjunction with each other to monitor entrants and others involved with the CSW in a centralized/ single point area, eliminating the need for attendants to be stationed outside each entrance of a Confined Space. This system monitors conditions inside the space, alerting entrants of changes in conditions, just the same as an attendant would. This approach uses available technology to see/hear/communicate to the entrants inside the space, ensuring personnel are safe as well as monitoring the conditions inside the space, alerting entrants when a dangerous condition occurs or is expected. Some stipulations need to be discussed and addressed before deciding to use this service: Exposures - System/service shall provide capability to audibly/visually monitor and oversee the space(s) included in job scope - Equipment needed for service shall not add additional access/egress restrictions or energy source exposures to the work scope without proper mitigation and contingency plans discussed and in place. Resources/Time - In using system/service, the proper competent safety critical personnel are in place to ensure safe completion of scope. - Begin discussions of using this service/system in advance so any pre-planning and resource allocation is given due diligence. - Specialist contractor providing service to system shall be evaluated to ensure they: - Can supply proper personnel/equipment to use and service system - Comply with Shell’s rules and requirements (Contractor Safety Management process, CSW CF manual, local, governmental, privacy, and Global laws and regulations). - Any data collected during the service is considered part of the work and is considered property of Shell (and its respective entities) and subject to local/ global/ governmental privacy regulations as well as record retention policies. - Training and competency assessments of service provided personnel shall be logged, reviewed, and confirmed before personnel use equipment/ gear for centralized monitoring. Personnel in critical roles shall be able to prove and demonstrate their competency. - System operators shall be present and attentive to monitoring the space/entrants while CSW is being conducted and shall only have one task: to monitor the equipment and entrants during CSW. Configuration of Equipment - The equipment must be certified to work in the area the CSW is taking place (i.e. Class 1 Zone/Division areas). If the equipment is not certified to operate in the area in which the activity is taking place, any risks relating to insufficiently certified equipment should be controlled by the use of the Permit to Work system/process. - The service provider shall discuss and provide a contingency plan for failures of equipment/critical spares of equipment becomes unavailable (discussed and agreed with the location). - The equipment shall be set up to replicate the required safety systems needed for safe completion of the CSW (i.e. gas detection and continuous gas monitoring as well as order of detection, ability to summon rescue team, ability to communicate changes in space conditions and require egress once conditions have escalated to an unsafe level, etc.). - The equipment/work party must provide a means of restricting access to the space as well as a means to login/logout entrants effectively. Management of CSW activities - The operator of the equipment shall ensure the system will be able to have oversight on all entrants, considering the configuration of the space or amount of activities going on inside the space. There shall also be a field based area attendant that monitors the CSW from deck/ worksite level to intervene when necessary as well as provide oversight and communication back to monitor operators. - Adequate and qualified personnel must be available so that the panel operator’s position is manned 100% of the time while entrants are in the confined space(s). Centralized CSW Monitoring is an “acceptable” means of conducting CSW using advanced technology, provided all prerequisite conditions are met and the system/operation of the system does not impact the safe completion of the activity(ies). Parameters should be set before use of this option to ensure all facets and risks of the CSW are known, discussed, and covered before commencement of the operation of the system. |  |  |
| Guidelines/Tools |  |  |  |  |
| Guideline 01: FPSO Stones Confined Space Supplemental Procedure | HSE0085-GL01.docx Use this guideline procedure for Confined Space ancillary information for FPSO Stones |  |  |  |
| CS Attendant/ Holewatch Form | Use this form to log/record the holewatches that are actively monitoring entries into CS. This form is to be retained with finalized work documents. |  |  |  |
| Gas Testing Form | Use this form to document the initial and periodic/ subsequent gas tests (min. every 30 mins). This form is to be retained with finalized work documents. |  |  |  |
| Personnel Login/Logout Form | Use this form to log/record all personnel entering and exiting the space. This is to be used as record to show all personnel have exited the space as well as said personnel are complete the task. This form is to be retained with finalized work documents. |  |  |  |
| L1/L2 Reclassification Form | Use this process and form to reclassify a confined space to a lower level due to circumstances/ configuration. This form and reclassification shall be completed annually and shall be kept on site for reference and review. Keep this form stored on site and accessible for audits/ investigations/ assessments of CSE |  |  |  |
| Confined Space Rescue Plan | Use this form to document the rescue plan, gear, personnel, and any other pertinent information to convey to the rescuers the process and steps to safely rescue entrant personnel. The process, along with critical contact information should be included in the rescue plan to ensure all parties understand the process and their roles and responsibilities. |  |  |  |
| Confined Space Start Work Checklist (Formerly Pre-Entry Checklist) | This form and actions are to be completed prior to entry into a L2 / L3 Confined Space. This form records all required actions of preparation and entry as well as requires OIM to witness final gas test before entry. This form is to be retained with finalized work documents. |  |  |  |