| Attention! Stair Tower Max Capacity 20 persons WIND LOADING THIS STAIR TOWER IS DESIGNED FOR A MAXIMUM WIND VELOCITY OF : 90 MPH ULTIMATE 70 MPH WITH A STRUCTURAL SAFETY FACTOR. 35 MPH WITH A 4:1 SAFETY FACTOR. ALL PERSONNEL MUST EVACUATE THIS STAIR TOWER IF SUSTAINED WIND VELOCITY EXCEEDS 35 MPH. STAIR TOWER MUST BE DISMANTLED IN ADVANCE OF PREDICTED WIND SPEEDS IN EXCESS OF 90 MPH STAIR TOWER PLANKING MUST BE REMOVED IN ADVANCE OF PREDICTED | MODULAR STAIR TOWER GENERAL NOTES: 1. GENERAL 1.1) USER/ERECTOR SHALL READ AND COMPLY WITH THE FOLLOWING: "SAFETY GUIDELINES FOR BRANDSAFWAY SYSTEMS SCAFFOLD". 1.2) THIS SCAFFOLD LAYOUT WAS DEVELOPED FROM LIMITED INFORMATION AND MAY VARY DUE TO ACTUAL FIELD CONDITIONS. CONTACT BRANDSAFWAY ENGINEERING FOR APPROVAL PRIOR TO MODIFYING. 1.3) THIS STAIR TOWER IS DESIGNED FOR A MAXIMUM LOADING OF (20) TWENTY WORKERS. WORKERS ARE TO BE EVENLY DISTRIBUTED THROUGHOUT THE HEIGHT OF THE STAIR TOWER WITH NOT MORE THAN TWO WORKERS ON EACH FLIGHT AT ONE TIME. 1.4) SIGNS MUST BE POSTED AT ENTRANCE AND EXIT OF TOWER ADVISING THE USER OF THE MAXIMUM NUMBER OF WORKERS ALLOWED ON THE STAIR TOWER. 1.5) THIS STAIR TOWER IS INTENDED TO PROVIDE TEMPORARY ACCESS AS REQURIED IN OSHA REGULATION 29 CFR 1926.451 |
| --- | --- |